HomeGuard Raspberry Pi Installation Guide

Flask dashboard, SQLite logging, ThingSpeak, Telegram, SenseHAT, and Camera setup

# 1. What This Guide Installs

This guide explains how to install and run HomeGuard on a Raspberry Pi. The Raspberry Pi scans the home network, checks unknown MAC addresses, logs data locally in SQLite, sends cloud readings to ThingSpeak, and shows everything in the Flask dashboard.

# 2. Hardware Needed

- Raspberry Pi 4 with Raspberry Pi OS installed

- MicroSD card, power supply, keyboard/monitor or SSH access

- Raspberry Pi SenseHAT

- Raspberry Pi Camera Module

- Internet connection for ThingSpeak and Telegram alerts

# 3. ThingSpeak Field Mapping

| ThingSpeak Field | HomeGuard Value | Example |
| --- | --- | --- |
| Field 1 | CPU Temperature (vcgencmd) | 47.7 |
| Field 2 | Humidity (0.0 - sensor issue) | 0.0 |
| Field 3 | Connected devices | 4 |
| Field 4 | Unknown devices | 1 |
| Field 5 | Alert status | 1 = Alert, 0 = Clear |

Note: SenseHAT humidity sensor was not detected due to a hardware/driver issue. CPU temperature is used instead via vcgencmd measure_temp. See Section 14 for details.

# 4. Prepare Raspberry Pi OS

Boot the Raspberry Pi and connect it to the same home network that will be monitored. Open Terminal, or connect using SSH. Update the package list and installed packages:

sudo apt update

sudo apt upgrade -y

# 5. Enable Camera and SenseHAT Support

Install the Raspberry Pi hardware packages and required system libraries:

sudo apt install -y sense-hat python3-picamera2 libcap-dev python3-rtimulib

Enable the SenseHAT overlay by adding it to the config file:

echo "dtoverlay=rpi-sense" | sudo tee -a /boot/firmware/config.txt

echo "dtparam=i2c_arm=on" | sudo tee -a /boot/firmware/config.txt

sudo reboot

After reboot, verify the SenseHAT LED works:

sudo python3 -c "from sense_hat import SenseHat; s = SenseHat(); s.clear(255,0,0); import time; time.sleep(3); s.clear(); print('SenseHAT OK')"

# 6. Install HomeGuard Project

Create the HomeGuard project folder on the Raspberry Pi:

mkdir ~/HomeGuard

cd ~/HomeGuard

Create the required files and folders:

touch app.py requirements.txt .env.example

mkdir templates static

Create a Python virtual environment and install the required packages:

python3 -m venv .venv

source .venv/bin/activate

pip install --upgrade pip

pip install -r requirements.txt

If you encounter a libcap error during installation, run:

sudo apt install -y libcap-dev

pip install -r requirements.txt

# 7. Configure .env File

Create and edit the .env configuration file:

nano .env

Recommended Raspberry Pi values:

HOMEGUARD_SCAN_INTERVAL=30

HOMEGUARD_UNKNOWN_THRESHOLD_SECONDS=120

HOMEGUARD_NETWORK_CIDR=192.168.1.0/24

HOMEGUARD_DEMO_MODE=false

THINGSPEAK_ENABLED=true

THINGSPEAK_CHANNEL_ID=your_channel_id

THINGSPEAK_WRITE_API_KEY=your_write_api_key

TELEGRAM_ENABLED=false

TELEGRAM_BOT_TOKEN=your_bot_token

TELEGRAM_CHAT_ID=your_chat_id

Important: Use your actual ThingSpeak Channel ID and Write API Key from the API Keys tab in your ThingSpeak channel settings.

# 8. Telegram Alert Setup

To receive Telegram alerts when an unknown device is detected:

- Open Telegram and search for BotFather (@BotFather with blue verified checkmark)

- Send /newbot and follow the steps to create a new bot

- Copy the bot token provided by BotFather

- Search for your new bot in Telegram and send it any message (e.g. 'hi')

- Find your chat ID by opening this URL in a browser: https://api.telegram.org/bot<YOUR_TOKEN>/getUpdates

- Copy the id value from the chat section in the response

Update your .env file with the Telegram details:

TELEGRAM_ENABLED=true

TELEGRAM_BOT_TOKEN=your_actual_bot_token

TELEGRAM_CHAT_ID=your_actual_chat_id

# 9. Run the Application

Start HomeGuard from the project folder. Note: sudo is required for network scanning:

cd ~/HomeGuard

source .venv/bin/activate

sudo python3 app.py

Also install required packages system-wide for sudo access:

sudo pip install flask python-dotenv requests scapy --break-system-packages

Open the dashboard from another device on the same network:

http://raspberry-pi-ip-address:5000

# 10. Camera Alert Rule

If an unknown MAC address remains connected for more than 2 minutes, HomeGuard will:

- Capture a camera image using Picamera2

- Send a Telegram alert if Telegram is enabled

- Flash the SenseHAT LED matrix red for 10 seconds

- Store the alert event and snapshot path in SQLite

To test this without a new device, comment out one MAC address from KNOWN_DEVICES in app.py and restart the app. The device will be treated as unknown and trigger an alert after 2 minutes.

# 11. Run Automatically on Boot

Create a systemd service file:

sudo nano /etc/systemd/system/homeguard.service

Paste the following content:

[Unit]

Description=HomeGuard Flask Monitoring System

After=network-online.target

[Service]

WorkingDirectory=/home/bounchun/HomeGuard

ExecStart=/usr/bin/python3 /home/bounchun/HomeGuard/app.py

Restart=always

User=root

[Install]

WantedBy=multi-user.target

Enable and start the service:

sudo systemctl daemon-reload

sudo systemctl enable homeguard

sudo systemctl start homeguard

sudo systemctl status homeguard

# 12. Testing Checklist

- Dashboard opens in the browser at http://<pi-ip>:5000

- Known devices appear as KNOWN (green badge)

- Unknown devices appear as UNKNOWN (red badge)

- ThingSpeak charts update after a scan cycle (Field 1 shows CPU temperature)

- Telegram alert is received for unknown devices

- SenseHAT turns red for 10 seconds when an unknown device is present

- Camera snapshot is saved after the 2 minute threshold

- Recent Alerts section shows alert history with View links for snapshots

# 13. Common Problems

| Problem | What to Check |
| --- | --- |
| Dashboard does not open | Check that sudo python3 app.py is running and port 5000 is not blocked. |
| No real devices found | Set HOMEGUARD_DEMO_MODE=false and check HOMEGUARD_NETWORK_CIDR. |
| ThingSpeak not updating | Verify the Write API Key matches the one in ThingSpeak API Keys tab. |
| No Telegram message | Check TELEGRAM_ENABLED=true, bot token, and chat ID in .env file. |
| Camera does not capture | Check camera cable connection and python3-picamera2 installation. |
| ModuleNotFoundError: scapy | Run: sudo pip install flask python-dotenv requests scapy --break-system-packages |
| Database is locked | Restart the service: sudo systemctl restart homeguard |
| SSH connection timed out after reboot | Wait 30-60 seconds for Pi to fully boot, then retry. |

# 14. Known Issues and Workarounds

## SenseHAT Humidity and Temperature Sensor

The SenseHAT humidity and temperature sensor may fail with OSError: Humidity Init Failed. This occurs when the I2C sensor addresses are not detected on the I2C bus.

Steps taken to debug:

- Verified I2C was enabled via raspi-config Interface Options

- Ran sudo i2cdetect -y 1 — SenseHAT sensor addresses not detected

- Added dtoverlay=rpi-sense and dtparam=i2c_arm=on to /boot/firmware/config.txt

- SenseHAT LED matrix works correctly and is used for alerts

- Only the humidity and temperature sensors are affected

Workaround: CPU temperature is read using vcgencmd measure_temp and sent to ThingSpeak Field 1. Humidity is logged as 0.0 until the sensor issue is resolved.

The get_sensehat_readings() function in app.py was updated to:

def get_sensehat_readings():

try:

import subprocess

result = subprocess.run(['vcgencmd', 'measure_temp'],

capture_output=True, text=True)

temp_str = result.stdout.strip()

temp = float(temp_str.replace("temp=", "").replace("'C", ""))

return round(temp, 1), 0.0

except Exception as e:

print(f"Temperature error: {e}")

return 0.0, 0.0

HomeGuard Installation Guide — Updated May 2026